Using MATLAB/Scilab, design a quarter-wave transformer to match a 40 Ω

load to a 75 Ω transmission line at 1.5 GHz. Determine the transformer

characteristic impedance and physical length.

Program code:

clc;

clear;

close;

// --------------------------------------------------

// QUARTER-WAVE TRANSFORMER DESIGN

// --------------------------------------------------

// Given parameters

Z0 = 75;              // Characteristic impedance of transmission line (ohms)

ZL = 40;              // Load impedance (ohms)

f0 = 1.5e9;           // Design frequency (Hz)

c = 3e8;              // Velocity of light (m/s)

// Transformer characteristic impedance

Zt = sqrt(Z0 * ZL);

// Wavelength at design frequency

lambda0 = c / f0;

// Quarter-wave transformer physical length

l = lambda0 / 4;

// Display calculated values

disp("==============================================");

disp("       QUARTER-WAVE TRANSFORMER");

disp("==============================================");

printf("Transmission line impedance = %.2f Ohm\n", Z0);

printf("Load impedance              = %.2f Ohm\n", ZL);

printf("Operating frequency         = %.2f GHz\n", f0/1e9);

printf("Wavelength                  = %.4f m\n", lambda0);

printf("Transformer impedance       = %.2f Ohm\n", Zt);

printf("Transformer length          = %.4f m\n", l);

printf("Transformer length          = %.2f cm\n", l*100);

// --------------------------------------------------

// INPUT IMPEDANCE CALCULATION

// Zin = Zt * (ZL + j*Zt*tan(beta*l))

//       / (Zt + j*ZL*tan(beta*l))

// --------------------------------------------------

f = linspace(0.5e9, 2.5e9, 1000);

lambda = c ./ f;

beta = 2 * %pi ./ lambda;

// Electrical length of transformer

theta = beta * l;

// Input impedance

Zin = Zt .* (ZL + %i*Zt.*tan(theta)) ./ ...

(Zt + %i*ZL.*tan(theta));

// Magnitude of input impedance

Zin_mag = abs(Zin);

// --------------------------------------------------

// GRAPH

// --------------------------------------------------

scf(1);

plot(f/1e9, Zin_mag, "b", "LineWidth", 2);

xgrid();

xlabel("Frequency (GHz)");

ylabel("|Z_{in}| (Ohm)");

title("Input Impedance of Quarter-Wave Transformer");

a = gca();

a.data_bounds = [0.5, 30; 2.5, 90];

// Mark design frequency

plot([1.5 1.5], [30 90], "r--", "LineWidth", 1.5);

plot(1.5, Z0, "ko");

legend(["Input impedance"; ...

"Design frequency"; ...

"75 Ohm matched impedance"], "in_upper_right");

disp("==============================================");

disp("Graph plotted successfully.");

disp("At 1.5 GHz, Zin is approximately 75 Ohm.");

disp("==============================================");

OUTPUT: